Recalibration of Air Flow Rate Using New Flow Meter

The new flow-meter has a scale of 0-100 in arbitrary units.  After the new flow-meter was incorporated into the system we decided to work to the 50-unit mark on the arbitrary scale, because that flow-rate centered the distribution of flies fairly well in about the middle of the tunnel, and was also a convenient intermediate mark to work to.  Then we tried to estimate the actual flow rate at 50 units.  This was difficult.  At first, we used an estimated flow rate of 9.1 CFM in converting the compartment counts into flying speeds.  Later on, we were able to get a more accurate calibration of the flow rate.  We found that at the 50-unit mark it was actually about 11 CFM.  At that point we switched to the estimate of 11 CFM, and we then went back and multiplied the previous calculated flying speeds by 1.21 to correct for the error.  For all the generations from 200 to 248, the flying speeds shown on the final graph are higher than the flying speeds recorded on the original data sheets, by a factor of 1.21.  Ever since generation 248, up to the present time, we have used the same 50-unit mark to adjust the flow rate, and we have used the same estimate of that flow rate at 11 CFM to calculate apparent flying speeds.