Digitizing Pointsets of Wing Outlines with ImageJ

To Start

Open ImageJ.  The toolbar comes up.

Go to File in the toolbar, and open a photo image file, from within ImageJ.

Select the polygon measuring tool from the ImageJ toolbar (the 3rd tool on the bar).

Go to Analyze, Set Scale.

Set the scale at 678 pixels/mm.  Check “global.”  Hit OK.

To Set Up

Go to Analyze, Set Measurements.

Below, check the box for Invert Y Coordinates.

Above, check also the boxes for Area, Centroid, Perimeter, Fit Ellipse, and Feret’s Diameter.

Uncheck all other boxes.

To Digitize Points

Each photo to be measured, must be opened from within ImageJ.

First enlarge the photo image to full screen.  Move the image so that the whole wing is visible.

Go around the wing image according to the standard predetermined pattern.

IMPORTANT:  Each major landmark must always have the same point number.

On the last point, to complete the set, either:  right click, or double click left.

This will create a connection back to the first point, completing the perimeter.

Now go back around the wing adjusting all the point locations.

Points can be against the edge of the photo, wherever the cursor appears as cross-hairs.

To Save Perimeter Points

Go to File, Save As, XY Coordinates.   (This must be done again for every wing…)

The destination folder should be the same as the photo image file and it will open automatically.

Save the XY coordinates for each wing in a separate file, with the same title as its photo, plus “pp.”

Use file type “All Files.”  Save as .txt. No new file extension is needed.

Each saved XY file will show up next to its own photo file, as a Notepad file.

To Save Area, Centroid (X,Y), Perimeter, Major & Minor Axes (and Angle) of Fitted Ellipse

Go to Analyze, hit Measure:  The values will all appear in Results box.

(In the Results box, go to File, Options, and make sure the boxes are unchecked for copying column headers and row numbers.  This only needs to be set the first time through.  The row number will still show but won’t transfer with data.)

Select and Cut the data from the box.

Open the photo folder that has the measured image (open it from the Desktop folder—e.g., KEN WINGS, or ALL NUVX—not through ImageJ).

Open the just-saved Notepad file of XY coordinates.

Paste in the additional data at the end of the file.

Save the data file again. Close it.

Kill the old wing image and go to File, Open for next wing.